QUESTION BANK CABIN BASIC

CAAV Banks For Cabin Repair

1. The cockpit is typically equipped with?

A. unadjustable seats for two crew members (the pilots), and two folding seats for flight observers

B. adjustable seats for two crew members (the pilots)

C. adjustable seats for two crew members (the pilots), and two folding seats for flight observers (*)

2. The flight compartment equipment and furnishings include these component groups on the flight deck?

A. Stowage compartments, Consoles.

B. Seats, Stowage compartments, Consoles. (*)

C. Seats, Entertainment equipment, Consoles.

3. The Captain and First Officer seats?

A. have only mechanical control

B. have only electrical control

C. have electrical and mechanical control (*)

4. Armrest on CAPT/FO seat can be?

A. folded back to a vertical position to enable easy access to the crew briefcase or console. (*)

B. fix to a vertical position to enable easy access to the crew briefcase or console.

C. folded back to a horizontal position to enable easy access to the crew briefcase or console.

5. The backrest of the CAPT/FO seats have:

A. The backrest of the FO seats have a mechanical control, The backrest of the CAPT seats fixed at upright position

B. The backrest of the CAP/FO seats fixed at upright position.

C. a mechanical control. (*)

6. Which is the correct sentence?

A. The seat also adjusts laterally and vertically and swivels to the left a maximum of 90 degrees; seat backrest recline adjusts within a range of about 45 degrees, and the armrests adjust to various positions

B. The seat also adjusts laterally and vertically and swivels to the left a maximum of 25 degrees; seat backrest recline adjusts within a range of about 90 degrees, and the armrests adjust to various positions

C. The seat also adjusts laterally and vertically and swivels to the left a maximum of 90 degrees; seat backrest recline adjusts within a range of about 25 degrees, and the armrests adjust to various positions (*)

7. Which is the correct sentence?

A. Actuation of the toggle switch located beside the horizontal adjustment switch provides 115/200-volt, 400-cycle, 3-phase ac power to an electric motor.

B. Actuation of the toggle switch located beside the vertical adjustment switch provides 28v DC power to an electric motor.

C. Actuation of the toggle switch located beside the vertical adjustment switch provides 115/200-volt, 400-cycle, 3-phase ac power to an electric motor. (*)

8. Which is the correct sentence with CAPT/FO seats?

A. The seats are equipped with 3 point safety harness.

B. The seats are equipped with 2 point safety harness.

C. The seats are equipped with 5 point safety harness. (*)

9. The rear panels are installed at the rear walls in the cockpit. These racks house typically:

A. the AC and DC electrical power centers, the primary circuit breakers and the power circuits, the system circuit breakers requiring no action in flight (*)

B. the DC electrical power centers, the primary circuit breakers and the power circuits, the system circuit breakers requiring no action in flight

C. the AC and DC electrical power centers, the primary circuit breakers and the power circuits, the system circuit breakers requiring action in flight

10. The passenger compartment is

A. in the upper fuselage section. It is between the partition of the flight compartment and the fwd pressure bulkhead

B. in the upper fuselage section. It is between the partition of the flight compartment and the aft pressure bulkhead (*)

C. in the upper fuselage section. It is beside the partition of the flight compartment and the aft pressure bulkhead

11. The cabin is divided into

A. utility areas, seating areas and galley areas

B. galleys, lavatories areas and seating areas

C. utility areas and seating areas (*)

12. The passenger seating area is between

A. the forward utility area and the aft utility area (*)

B. the forward galley area and the aft galley area

C. the forward lavatory area and the aft galley area

13. Which is the correct sentence?

A. Partitions and curtains divide the utility areas

B. Partitions and curtains divide the seating areas.

C. Partitions and curtains divide the utility areas from the seating areas. (*)

14. The equipment and furnishings which are installed in the passenger seating area are?

A. passenger seats, cabin attendant seats, linings and furnishings that cover the compartment structure, passenger service/information units, textile floor coverings.

B. passenger seats, cabin attendant seats, linings and furnishings that cover the compartment structure, overhead stowage compartments for stowage of passenger carry-on baggage and other equipment, passenger service/information units.

C. passenger seats, cabin attendant seats, linings and furnishings that cover the compartment structure, overhead stowage compartments for stowage of passenger carry-on baggage and other equipment, passenger service/information units, textile floor coverings. (*)

15. The equipment and furnishings which are installed in the utility areas are?

A. Passenger Service Units (PSUs) are installed at passenger and attendant locations throughout the cabin, curtains and partitions are used to divide the utility areas and the seating sections, ancillary equipment has different stowage units, nontextile floor coverings, galleys, lavatories

B. cabin attendant seats, Passenger Service Units (PSUs) are installed at passenger and attendant locations throughout the cabin, curtains and partitions are used to divide the utility areas and the seating sections, ancillary equipment has different stowage units, nontextile floor coverings, galleys, lavatories (*)

C. cabin attendant seats, Passenger Service Units (PSUs) are installed at passenger and attendant locations throughout the cabin, curtains and partitions are used to divide the utility areas and the seating sections, ancillary equipment has different stowage units, galleys, lavatories

16. Additional equipment also installed in the cabin is?

A. emergency escape slide/slide rafts, first aid equipment, miscellaneous emergency equipment (*)

B. emergency escape slide/slide rafts, miscellaneous emergency equipment.

C. emergency escape slide/slide rafts, first aid equipment, miscellaneous emergency equipment, loosen emergency

17. Which is the correct sentence?

A. The passenger seats are installed on the seat tracks of the stringer floor structure.

B. The passenger seats are installed on the seat tracks cover of the cabin floor structure.

C. The passenger seats are installed on the seat tracks of the cabin floor structure. (*)

18. Which is the correct sentence?

A. The reclining back rest, the armrests, the seat belts and the seat pan are attached to the seat assembly

B. The reclining back rest, the armrests, the seat belts and the seat pan are attached to the seat frame assembly (*)

C. The armrests, the seat belts and the seat pan are attached to the seat frame assembly

19. The backrest and seat cushion pads are?

A. covered with textile covers.

B. covered with flame resistant covers.

C. covered with heat resistant covers. (*)

20. The Passenger Seat consists of a framework assembly with?

A. an upholstered backrest assembly, a folding table, seat belts, an upholstered armrest and, a pocket.

B. an upholstered seatpan, an upholstered backrest assembly, a folding table, an upholstered armrest and, a pocket.

C. an upholstered seatpan, an upholstered backrest assembly, a folding table, seat belts, an upholstered armrest and, a pocket. (*)

21. Which is the correct sentence?

A. The upholstered seatpan is installed on the seat framework. It is not foldable or adjustable. (*)

B. The upholstered seatpan is installed on the seat framework. It is foldable and adjustable.

C. The upholstered seatpan is installed on the seat framework. It is non foldable and adjustable.

22. The upholstered backrest assembly of each seat can be?

A. pneumatically reclined backwards until it hits an adjustable mechanical stop.

B. mechanically reclined backwards until it hits an adjustable mechanical stop.

C. hydraulically reclined backwards until it hits an adjustable mechanical stop. (*)

23. The armrests between the seats?

A. can not be folded up.

B. can be folded up. (*)

C. can not be stowed.

24. Which is the correct sentence?

A. The reclining control button for the backrest and an ashtray are installed in the armcap.

B. The reclining control button for the backrest and an ashtray are installed in the body armrest

C. The reclining control button for the backrest and an ashtray are installed in the armrest (*)

25. Which is the correct sentence?

A. A folding table is installed in the rear of each backrest. When not in use, it is kept in a recess in the backrest by means of a latch. (*)

B. A folding table is installed in the front of each backrest. When not in use, it is kept in a recess in the backrest by means of a latch.

C. A folding table is installed in the rear of each backrest. When in use, it is kept in a recess in the backrest by means of a latch.

26. In-armrest tables are?

A. installed in the seats, located just behind partitions, galleys.

B. installed in the seats, located just behind partitions, galleys, lavatories and curtains.

C. installed in the seats, located just behind partitions, galleys or lavatories (*)

27. If an armrest with a recline mechanism is missing?

A. the seat can still be occupied if it is locked in the full downright position

B. the seat can not be occupied if it is locked in the full upright position

C. the seat can still be occupied if it is locked in the full upright position (*)

28. The attendant seats attach to?

A. the seat tracks on the floor (*)

B. the beam on the floor

C. the stringer on the floor

29. The attendant seat has?

A. a seat back that folds up when not in use.

B. a seat pan that folds up when not in use. (*)

C. a seat pan that folds down when not in use.

30. The cabin attendant seats?

A. are installed in the areas of the passenger/crew doors.

B. are installed in the areas of the emergency exits

C. are installed in the areas of the passenger/crew doors and emergency exits (*)

31. The wall mounted cabin-attendant seats are installed on?

A. the lavatory walls, the galley walls, the stowage walls (*)

B. the lavatory walls, the galley walls

C. the lavatory walls, the galley walls, the stowage walls The divider walls

32. The cabin-attendant seats have?

A. a seat frame, a backrest with a cushion, a seatpan with a cushion, a headrest with a cushion, a stowage compartment.

B. a seat frame, a backrest with a cushion, a seatpan with a cushion, a headrest with a cushion, a restraint system.

C. a seat frame, a backrest with a cushion, a seatpan with a cushion, a headrest with a cushion, a restraint system, a stowage compartment. (*)

33. The restraint system is?

A. divided into a seat belt assy and a shoulder harness (*)

B. divided into a seat belt assy and inertia reels

C. divided into a shoulder harness and inertia reels

34. The shoulder harness uses?

A. inertia reels to manually pull it in when the seat is in use

B. inertia reels to automatically pull it in when the seat is not in use (*)

C. harness to automatically pull it in when the seat is not in use

35. A stowage compartment?

A. below the seat pan or inside the headrest can be used to stow miscellaneous equipment

B. below the seat pan or beside the headrest can be used to stow emergency equipment

C. below the seat pan or inside the headrest can be used to stow emergency equipment (*)

36. The cabin attendant seats have?

A. a 5-point restraint system to prevent injuries to the cabin attendants in an emergency

B. a 4-point restraint system to prevent injuries to the cabin attendants in an emergency (*)

C. a 2-point restraint system to prevent injuries to the cabin attendants in an emergency

37. Which is the correct sentence?

A. The EASA OPS 1 requires a safety belt with safety belt for each flight crew seat and for any seat alongside a pilots seat incorporating a device which will automatically restrain the occupants torso in the event of rapid deceleration

B. The EASA OPS 1 requires a safety belt with shoulder harness for each flight crew seat and for any seat alongside a pilots seat incorporating a device which will manually restrain the occupants torso in the event of rapid deceleration

C. The EASA OPS 1 requires a safety belt with shoulder harness for each flight crew seat and for any seat alongside a pilots seat incorporating a device which will automatically restrain the occupants torso in the event of rapid deceleration (*)

38. All safety belts with shoulder harness must have?

A. a double point release.

B. a triple point release.

C. a single point release. (*)

39. The galleys are?

A. Seller Furnished Equipment

B. Buyer Furnished Equipment (*)

C. Lower Furnished Equipment

40. Galleys are?

A. either fixed or movable, but all of them are mounted on the standard seat tracks or hard points and ceiling attachments. (*)

B. movable, but all of them are mounted on the standard seat tracks or hard points and ceiling attachments.

C. either fixed or movable, but all of them are mounted on the standard seat tracks or soft points and ceiling attachments.

41. The galleys give?

A. rest areas that can hold food and where flight attendants can prepare meals

B. work areas that can hold waste and where flight attendants can prepare meals

C. work areas that can hold food and where flight attendants can prepare meals (*)

42. The galley can hold?

A. full size trolleys and containers

B. full and/or half size trolleys and containers (*)

C. full and/or half size trolleys

43. There are optional galley configurations of?

A. wet or hot units

B. hot or cold units

C. wet or dry units (*)

44. The dry galleys are?

A. not connected to the aircraft systems such as ventilation or potable water (*)

B. not connected to the aircraft systems such as ventilation or potable water or electrical

C. not connected to the aircraft systems such as ventilation

45. The wet galleys are connected to?

A. the water and waste system, the air extraction system, the electrical power system.

B. the water and waste system, the main air cabin ventilation system and the electrical power system.

C. the water and waste system, the air extraction system, the main air cabin ventilation system and the electrical power system. (*)

46. Potable water for the galleys comes from?

A. the water tanks aft of the bulk cargo compartment (*)

B. the waste tanks aft of the bulk cargo compartment

C. the water tanks aft of the fwd cargo compartment

47. The galley liquid and small solid waste?

A. goes to the water drain system.

B. goes to the water system.

C. goes to the vacuum waste system. (*)

48. These devices in the fixed galley use electrical power are?

A. Area lights, Emergency lights, Information signs, Galley inserts.

B. Area lights, Call lights, Emergency lights, Information signs, Galley inserts. (*)

C. Area lights, Call lights, Emergency lights, Galley inserts.

49. The GCU (Galley Cooling Unit) is?

A. a part of the galley refrigeration system (GRS) which supplies cold air to the galley foodcarts and galley ventilation.

B. a part of the galley refrigeration system (GRS) which supplies cold air to the galley foodcarts, beverage.

C. a part of the galley refrigeration system (GRS) which supplies cold air to the galley foodcarts. (*)

50. The lavatory/galley ventilation system is a draw-through system that uses 2 fans?

A. which are in the bulk cargo area and the air goes out of the airplane near the aft pressurization outflow valve. (*)

B. which are in the bulk cargo area and the air recircle of the airplane near the aft pressurization outflow valve.

C. which are in the bulk cargo area and the air goes out of the airplane near the aft pressurization outboard valve.

51. Each galley has standard inserts that hold galley equipment. There are?

A. 3 sizes of inserts.

B. 2 sizes of inserts.

C. 4 sizes of inserts. (*)

52. Size 4 inserts usually hold these types of galley equipment?

A. Bun warmers, Induction ovens, Oven.

B. Bun warmers, Induction ovens. (*)

C. Induction ovens, Oven.

53. Water flows?

A. automatically from the aircraft potable-water system to the water boilers. (*)

B. from the aircraft potable-water system to the water boilers by shut off valve

C. automatically from the aircraft potable-waste system to the water boilers.

54. The boiler has?

A. A level sensor, A temperature sensor, A pressure relief valve.

B. A level sensor, The heating element assemblies, A pressure relief valve.

C. A level sensor, A temperature sensor, The heating element assemblies, A pressure relief valve. (*)

55. The water heater has?

A. A water faucet, A receptacle for an electrical connector, A water connection. (*)

B. A receptacle for an electrical connector, A water connection.

C. A water faucet, A receptacle for an electrical connector.

56. Steam oven?

A. is to make the cooked meals hot.

B. to make the pre-cooked meals cold

C. is to make the pre-cooked meals hot. (*)

57. The steam oven has?

A. A control panel, A receptacle for an electrical connector.

B. A control panel, A receptacle for an electrical connector, A water connection. (*)

C. A control panel, A receptacle for an electrical connector, A water connection, Water outlet valve.

58. The espresso maker?

A. prepares hot espresso and cappuccino (*)

B. prepares hot espresso and americano

C. prepares cold espresso and cappuccino

59. The espresso maker includes?

A. an frothing mechanism to prepare the beverages with steamed milk.

B. an auto-frothing mechanism to prepare the beverages with cold milk.

C. an auto-frothing mechanism to prepare the beverages with steamed milk. (*)

60. The refrigerator keeps?

A. food and beverages frost.

B. food and beverages warm

C. food and beverages cold (*)

61. Lavatories give?

A. passengers and crew personal hygiene accommodations in the passenger cabin (*)

B. crew personal hygiene accommodations in the passenger cabin

C. passengers and crew personal hygiene accommodations in the seat areas.

62. Lavatories can be?

A. at fixed locations or at different locations in the outboard adaptable zones.

B. at fixed locations in the inboard and outboard adaptable zones.

C. at fixed locations or at different locations in the inboard and outboard adaptable zones. (*)

63. These are typical lavatory interfaces?

A. Lights and speakers, Smoke detector and fire extinguisher, Water and waste system, Oxygen system, Ventilation system.

B. Electrical power system, Lights and speakers, Smoke detector and fire extinguisher, Water and waste system, Oxygen system, Ventilation system. (*)

C. Electrical power system, Lights and speakers, Smoke detector and fire extinguisher, Water and waste system, Ventilation system.

64. The door of each lavatory can be?

A. unlocked from the passenger compartment side.

B. can be unlocked from inside. (*)

C. unlocked from the passenger compartment side.

65. Each lavatory is attached?

A. to the seat tracks on the floor. It is also attached to the airplane structure with support frames.

B. to the beam on the floor. It is also attached to the airplane structure with support rods

C. to the seat tracks on the floor. It is also attached to the airplane structure with support rods (*)

66. The washbasin cabinet has a door that gives access to?

A. the washbasin drain lines, the potable water faucet supply lines, the potable heater, the waste container, the waste container fire extinguisher. (*)

B. the washbasin drain lines, the potable water faucet supply lines, the potable heater, the waste container.

C. the potable water faucet supply lines, the potable heater, the waste container, the waste container fire extinguisher.

67. The ashtrays are installed?

A. internally on each lavatory.

B. externally on each lavatory.

C. internally and externally on each lavatory. (*)

68. A baby nursing table is installed in each lavatory?

A. For use, unlock and fold up the nursing table on to its support, which is attached to a lavatory wallpanel.

B. For use, unlock and fold down the nursing table on to its support, which is attached to a lavatory wallpanel. (*)

C. For use, unlatch and fold down the nursing table on to its support, which is attached to a lavatory wallpanel.

69. The service cabinet has?

A. an individual air outlet/ a loudspeaker (passenger address system)/ a dispenser for towels, sickbags, etc./ a razor socket/ a call button.

B. an individual air outlet/ a loudspeaker (passenger address system)/ a dispenser for towels, sickbags, etc.,/ a lighted sign RETURN TO SEAT and/or NO SMOKING/ a call button.

C. an individual air outlet/ a loudspeaker (passenger address system)/ a dispenser for towels, sickbags, etc.,/ a razor socket/ a lighted sign RETURN TO SEAT and/or NO SMOKING/ a call button. (*)

70. The Cabin Intercommunication Data System (CIDS) provides an interface between cockpit/cabin crew and cabin systems?

A. an interface between cockpit/cabin crew and cabin systems (*)

B. an interface between cockpit/cabin crew and ground

C. an interface between cockpit/cabin crew and passenger

71. Some of the features that can be included in cabin systems are?

A. Passenger Address/ Cabin Interphone/ Flight Attendant Calling/ Telephone, Fax and Internet Service.

B. Passenger Address/ Cabin Interphone/ Flight Attendant Calling/ Audio and Video Entertainment/ Telephone, Fax and Internet Service. (*)

C. Passenger Address/ Flight Attendant Calling/ Audio and Video Entertainment/ Telephone, Fax and Internet Service.

72. A cabin intercommunication data system is used by?

A. flight attendants and passengers

B. flight attendants, pilots, passengers and technical staff

C. flight attendants, pilots, and passengers (*)

73. Which is the correct sentence?

A. A cabin network service is a digital system that is typically hosted on a server within the internet

B. A cabin network service is a digital system that is typically hosted on a server outside the aircraft

C. A cabin network service is a digital system that is typically hosted on a server within the aircraft (*)

74. Which is the correct sentence?

A. The DUs are connected to three electronics units (EUs) located on the airplane information management system (AIMS) rack in the main equipment center

B. The DUs are connected to two electronics units (EUs) located on the airplane information management system (AIMS) rack in the secondary equipment center

C. The DUs are connected to two electronics units (EUs) located on the airplane information management system (AIMS) rack in the main equipment center (*)

75. The Light cover is used for?

A. diffusing the sidewall light (*)

B. protecting the sidewall light

C. decorating purpose only

76. The spacer panel has?

A. no window cutout (*)

B. only one window cutout

C. more than two window cutout

77. The spacer panel fills?

A. the space between two typical sidewall panels

B. the space between a monument and a typical sidewall panel (*)

C. the space where has emergency exit

78. What is the correct sentence?

A. The decompression grid dado-panel supplies more venting between the cabin and the cargo compartments if there is a rapid decompression (*)

B. The decompression grid dado-panel prevents venting between the cabin and the cargo compartments if there is a rapid decompression

C. The decompression grid dado-panel supplies more venting between the cabin and the air if there is a rapid decompression

79. The decompression grid dado-panel are attached?

A. to the structure and to the floor angles (*)

B. to the sidewall panel and to the floor angles

C. to the structure, to the sidewall panel and to the floor angles

80. Each frame of the decompression grid dado-panel has?

A. two flaps (*)

B. one flap

C. one or two flaps

81. The decompression grid dado-panel flaps are put back their initial positions after a rapid decompression?

A. by the springs (*)

B. by the hydraulic cylinder

C. by panel gravity

82. What is the correct sentence?

A. The primary function of the OHSCs is to keep the passenger hand luggage, emergency equipment and other equipment. (*)

B. The primary function of the OHSCs is to keep the passenger baggage, emergency equipment and other equipment.

C. The primary function of the OHSCs is to keep the passenger hand luggage, emergency equipment.

83. What is the main purpose of partition and curtain?

A. Dividing the cabin into the passenger areas and the utility areas.

B. Dividing the passenger areas into different classes

C. Both 1 and 2 (*)

84. The curtain system may has?

A. the curtain and the curtain rail

B. the curtain and the header with the curtain rail (*)

C. the curtain and the header

85. The curtain header where it is possible to install?

A. safety sign

B. placards

C. signs (for example, exit sign) and placards to obey the safety regulations (*)

86. The partitions are made of?

A. Honeycomb sandwich panels with decor covers. (*)

B. Honeycomb sandwich panels and painted coating

C. Composite panels

87. The edges of the partition is protected by?

A. trim cover (*)

B. making smooth surface

C. decorative laminate

88. There are two different types of partitions?

A. Full-height partitions and Under-bin partitions (*)

B. Business partitions and Economy partitions

C. Fixed partitions and Moveable partitions

89. The bottom end of full-height partitions are attached to the seat rails?

A. The top sides of the full-height partitions are attached to the structure. (*)

B. The top sides of the full-height partitions are attached to the overhead stowage compartment

C. The top sides of the full-height partitions are attached to the passenger service-unit channels.

90. The bottom end of under-bin partitions are attached to the seat rails?

A. The top sides of the under-bin partitions are attached to the overhead stowage compartment

B. The top sides of the full-height partitions are attached to the structure

C. The top sides of the under-bin partitions are attached to the structure (*)

91. The floor covering is for?

A. the comfort of the passengers and for the protection of the floor structure.

B. the comfort of the passengers and for the protection of the floor panels

C. the comfort of the passengers and for the protection of the floor panels and the floor structure. (*)

92. The textile carpet are kept in smooth surface by?

A. attached to floor by double-sided adhesive

B. by seat rail cover

C. attached to floor by double-sided adhesive and seat rail cover (*)

93. MYLAR is a self-adhesive polyester foil, which is attached to the floor panels in the wet areas of the cabin, for example door areas. This material?

A. gives more protection from liquids to the floor panels and the floor structure. (*)

B. makes a smooth surface with the non textile floor covering sheets.

C. increases the adhesive strength.

94. Which sentence is correct?

A. The emergency equipment is installed for the safety of the passengers and the crew, if an emergency occurs on the aircraft.

B. The emergency equipment is installed for the safety of the passengers, if an emergency occurs on the aircraft.

C. The emergency equipment is installed for the safety of the crew, if an emergency occurs on the aircraft. (*)

95. The emergency equipment list (EEL) typically contains?

A. a graphical location of emergency equipment on board the aircraft along with a description and part number associated with each item accordingly (*)

B. a graphical location of emergency equipment on board the aircraft along with part number associated with each item accordingly

C. location of emergency equipment on board the aircraft along with a description and part number associated with each item accordingly

96. We can find the part number associated with each emergency equipment with?

A. IPC

B. IPC and EEL

C. EEL (*)

97. If there is an emergency, the cockpit crew can go out of the cockpit by through the cockpit escape hatch in the cockpit ceiling. They can use?

A. ladder

B. Rope (*)

C. Parachute

98. The cockpit escape hatch is installed in the cockpit ceiling?

A. above the pilot seat.

B. above the co-pilot seat.

C. above the third occupant seat. (*)

99. The cockpit escape hatch?

A. is only opened from inner side of the aircraft (*)

B. is only opened from outer side of the aircraft

C. is only opened from inner side or outer side of the aircraft

100. The cabin escape facilities permit?

A. quick evacuation for the passengers and the crew in an emergency. (*)

B. quick evacuation for the passengers in an emergency.

C. quick evacuation for the crew in an emergency.

101. The inflation of the escape slides installed at all cabin doors is?

A. automatic if the door is opened with the Emergency Control Handle (ECH) in the ARMED mode (*)

B. automatic if the door is opened with the Emergency Control Handle (ECH) in the DIS-ARMED mode

C. manual if the door is opened with the Emergency Control Handle (ECH) in the ARMED mode

102. The inflation procedure of door escape slide takes approximately?

A. 3.5 s (*)

B. 4.5 s

C. 4 s

103. The pressure decrease in a reservoir of slide could be realized by?

A. the CIDS (*)

B. checking the pressure at the direct reading gage

C. the CIDS and checking the pressure at the direct reading gage

104. The CIDS caution light on the FWD Attendant Panel will come on; an illuminated system status message 'SLIDES PRESS LOW' on the Programming and Test Panel (PTP) will come on?

A. That means: the pressure in slide reservoir decreases (*)

B. That means: the pressure in slide reservoir increases

C. The slide deployed

105. A inflation system of door escape slide has?

A. an inflation reservoir (Nitrogen/Carbon Dioxide); a valve/regulator assembly and an aspirator. (*)

B. an inflation reservoir (Oxygen/Carbon Dioxide); a valve/regulator assembly and an aspirator.

C. an inflation reservoir (Nitrogen/Oxygen); a valve/regulator assembly and an aspirator.

106. Door Escape Facilities. Each escape facility has?

A. a pack assembly; an inflatable assembly; an inflation system; a decorative cover (*)

B. a pack assembly; an inflatable assembly; an inflation system.

C. a pack assembly; an inflatable assembly; an inflation reservoir; a decorative cover

107. During transport and removal/installation of any escape slide with a pressure reservoir?

A. the safety lockpin must be installed (*)

B. the safety cover must be installed

C. the lockpin must be removed

108. To make the slide serviceable, after the slide installation is completed?

A. the lockpin must be removed (*)

B. the lockpin must be installed

C. the lockpin must be installed and lockwired

109. The first aid equipment is used by crew to give medical aid?

A. to passengers who become ill or have small injuries during flight.

B. to passengers or to crew members who become ill or have small injuries during flight. (*)

C. to passengers who have request during flight.

110. How many typical type of first aid equipment?

A. 2

B. 3 (*)

C. 4

111. The typical first aid equipments are?

A. First Aid kit; Emergency Medical Kit; External Defibrillator (*)

B. First Aid kit; Emergency Medical Kit; Demo Kit

C. First Aid kit; Emergency Medical Kit; External Defibrillator; Demo Kit

112. The doctor kit is?

A. the kit contains medicines and equipment to help passengers or crew members who are badly injured or taken dangerously ill. (*)

B. the kit to help passengers or to crew members who become ill or have small injuries during flight.

C. the kit just be used by doctor on board

113. The life raft is inflated with?

A. high pressure air or nitrogen (N2) through an integral inflation System (*)

B. high pressure air or carbon dioxide (CO2) through an integral inflation System

C. high pressure nitrogen (N2) mixed with carbon dioxide (CO2) through an integral inflation System

114. The number of life rafts installed, and their location in the cabin, may vary in accordance with?

A. the aircraft type and the number of passengers accommodated (*)

B. the aircraft type and the number of passengers on board

C. the aircraft type and the number of crews and passengers accommodated

115. A typical life raft has an approved overload capacity of?

A. 0.5 (*)

B. 0.1

C. 0.3

116. Life Raft Lights?

A. are powered by a lithium dry cell battery (*)

B. are powered by a Ni-Cd battery

C. are powered by solar energy

117. There are different types of life vests?

A. Crew life vests/ Passenger (PAX) life vests/ Infant life vests/ Demonstration life vest. (*)

B. Crew life vests/ Passenger life vests/ Demonstration life vest.

C. Crew life vests/ Passenger life vests/ Infant life vests.

118. The life vests are inflated by?

A. pulling on the inflator of the carbon dioxide cylinder or oral inflation (*)

B. pulling on the inflator of the carbon dioxide cylinder

C. oral inflation

119. Which sentence is correct?

A. The hood is vacuum sealed and is installed in a protective container. (*)

B. The hood is sealed and is installed in a protective container.

C. The hood is sealed and is covered by fire-resistant material

120. PBE could be?

A. reused after recharging

B. intended for one time use only

C. intended for one time use only and is not rechargeable. (*)

121. Each supplementary survival kit contains these items?

A. A watermaker osmosis pump/ A parachute distress signal/ Water purification tablets/ Glucose tablets and Emergency Locator Transmitter (ELT) System (*)

B. A watermaker osmosis pump/ A parachute distress signal/ Water purification tablets/ Glucose tablets

C. A watermaker osmosis pump/ Water purification tablets/ Glucose tablets and Emergency Locator Transmitter (ELT) System

122. The flash lights?

A. use aircraft electrical power to charge battery

B. can operate without the aircraft electrical system (*)

C. can operate with solar energy

123. The flash lights?

A. The flashlights come on automatically when you remove them from the brackets and they go off when you put them back into the brackets. (*)

B. The flashlights come on automatically when you remove them from the brackets and they go off when you turn them off

C. The flashlights come on when you remove them from the brackets and turn on and they go off when you turn them off

124. The flash light bracket is used for?

A. Holding the flashlight only

B. Holding the flashlight and we can examine the condition of the battery (*)

C. examining the condition of the battery

125. How to test the flashlights when they are in bracket?

A. push the button switch on the bracket and examine the condition of the battery pack by check an LED indication (*)

B. push the button switch on the bracket and check flashlights come on or not

C. push the button switch on the flashlights and check flashlights come on or not

126. We must turn on the flash light by?

A. pushing the button switch on the bracket

B. pushing the button switch on the flashlight

C. The flashlights come on automatically when you remove them from the brackets and they go off when you turn them off (*)

127. The fireproof gloves?

A. protect the hands from heat and fire during an emergency (*)

B. protect the hands from heat during working with oven

C. protect the hands from fire during an emergency

128. The fireproof gloves are?

A. made of cotton-lined aramid fiber (*)

B. made of carbon fiber material

C. made of Kevlar material

129. The special safety belts in the aircraft are?

A. Safety extension-belt; demonstration safety belt; infant safety belt. (*)

B. Safety extension-belt; infant safety belt.

C. Safety extension-belt; demonstration safety belt.

130. The safety belt extensions?

A. increase the length of the safety belts (*)

B. is used to show the passengers how to use their safety belts

C. give protection to the infants who must sit on their parents' laps.

131. The demonstration safety belts?

A. increase the length of the safety belts

B. is used to show the passengers how to use their safety belts (*)

C. give protection to the infants who must sit on their parents' laps.

132. The infant safety belts?

A. increase the length of the safety belts

B. is used to show the passengers how to use their safety belts

C. give protection to the infants who must sit on their parents' laps. (*)

133. Defibrillators is used?

A. to give medical aid to passengers who have a heart attack during flight (*)

B. to give medical aid to passengers who have an allergy during flight

C. to give medical aid to passengers who have a high blood pressure during flight

134. The crash axes make it possible to?

A. get access to the cabin or go out of the cabin if there is an emergency, also get access to the aircraft components behind the linings or the blinders if there is a fire (*)

B. get access to the cabin or go out of the cabin if there is an emergency

C. get access to the aircraft components behind the linings or the blinders if there is a fire

135. The function of the megaphones is?

A. to increase the voice of the crew during an emergency in the aircraft (*)

B. to increase the voice of the crew during a PA announcement

C. to increase the voice of the crew during a PA announcement

136. Each megaphone has?

A. a battery pack for power supply and has no connection to the electrical system of the aircraft (*)

B. a battery pack for power supply connected to the electrical system of the aircraft

C. a battery pack charged by the electrical system of the aircraft

137. The megaphone is operated?

A. Push a button on its handle (*)

B. Push a button on its body

C. automatically when it is removed from the bracket.

138. The ELT transmitter system is operated by?

A. high powered batteries installed in the ELT transmitter cylinder housing (*)

B. solar energy

C. a portable dynamo

139. The ELT transmitter system has?

A. an ELT transmitter; a programming box assembly; a remote control unit; an external antenna (*)

B. an ELT transmitter; a programming box assembly; an external antenna

C. an ELT transmitter; a remote control unit; an external antenna

140. The remote control panel permits the manual operation and self test (BITE) of the ELT transmitter?

A. It is installed on the flight compartment overhead panel, and is connected to the programming box (*)

B. It is installed near on Flight Attendant Panel

C. It is installed on the center pedestal

141. The portable fire extinguishers is used to manually extinguish fires?

A. on board (*)

B. in cargo

C. in engine

142. UPK is?

A. equipped for use of cabin crew members in managing incidents of ill health associated with a case of suspected communicable disease, or in the case of illness involving contact with body fluids (*)

B. used to give medical aid to passengers or to crew members who become ill or have small injuries during flight.

C. designed to provide basic first aid supplies for minor injuries, such as cuts, scrapes, and burns, and to be used by passengers or crew members in case of emergencies.

143. During maintenance of Airbus 320/321 pilot seat, when actuator is disconnected from seat wiring, you must install a protection cap or adhesive tape to?

A. prevent electro-static shock (*)

B. prevent connecting pin corrosion, broken.

C. prevent dust, contamination.

144. The maximum times of dry cleaning cycles applied for fabric seat cover of Airbus 320/321 pilot seat is?

A. 10 (*)

B. 3

C. 5

145. Which way is correct for cleaning sheep skin seat cover of Airbus 320/321 pilot seat?

A. Clean the covers with an air cleaning process. Lightly brush the local area of the stain with a soft brush soaked with soapy water. (*)

B. Immerse the sheep-skin cover in soapy water

C. Dry cleaning

146. During maintenance of Airbus 320/321 pilot seat, the mechanical fault isolation of restraint system and its sub-components will be referred to?

A. CMM (*)

B. Restraint system CMM

C. related AMM task

147. Related to Airbus 320/321 pilot seat maintenance, damages on riveted or welded junctions must only are repaired?

A. by the seat manufacturer (*)

B. I.A.W CMM instruction

C. by trained staff

148. When do functional check of Airbus 320/321 pilot seat. Make 2 cycles of the horizontal and vertical movement (in mechanical and energized control). The seat movement - functions are good if?

A. there are no unexpected noise and no irregular movement as well as energized and mechanical control. (*)

B. there are no irregular movement.

C. there are no unexpected noise.

149. Do the functional check of Airbus 320/321 pilot seat, the vertical/horizontal movement with energized control is stopped?

A. by an integrated switch in the reduction gear actuator (*)

B. by stop operating switch

C. by an integrated switch in the reduction gear actuator or by stop operating switch

150. If you get a Airbus A320/A321 seat and found that storage period being superior to six months, the seat must be applied?

A. a preventive control process before the return to service including control operation for electrical seats (*)

B. a preventive control process before the return to service including control operation for mechanical seats

C. a preventive control process before the return to service including control operation for electrical/mechanical seats

151. The check of electrical system of Airbus A320/321 pilot/co-pilot seat includes?

A. Activation of the switches, the electrical horizontal and vertical function of the seat (by opening the OPEN/CLOSE function of C/B); lighting of the bulbs and their attachment on the seat bottom; the grounding points and the bonding braids (*)

B. Activation of the switches, the electrical horizontal and vertical function of the seat (by opening the OPEN/CLOSE function of C/B); lighting of the bulbs; the grounding points and the bonding braids.

C. Activation of the switches, the electrical horizontal and vertical function of the seat (by opening the OPEN/CLOSE function of C/B); lighting of the bulbs and their attachment on the seat bottom; the bonding braids.

152. Related to pilot/co-pilot seat functional check, when you pull upward the seat backrest control handle, which result shows that the recline backrest function is good?

A. The seat backrest automatically comes back to forward position in two seconds.

B. The seat backrest automatically comes back to forward position

C. The seat backrest comes back to forward position by hand-pulling

153. A pilot/co-pilot seat have good recline function if you pull upward recline control handle, the backrest could?

A. Automatically come back to forward position in two seconds and have good synchronization of seatbacks locks. (*)

B. Automatically come back to forward position and have good synchronization of seatbacks locks

C. Automatically come back to forward.

154. If the pilot/co-pilot seat backrest does not stay in locked position after handle release, the probable cause may be?

A. One of the locks misadjusted or damaged (*)

B. defective gas cylinder

C. broken spring of handle

155. The recline control handle pivots but the seat backrest does not spring back to its initial position, the probable cause may be?

A. One of the locks misadjusted or damaged

B. One of the locks misadjusted or damaged and/or defective gas cylinder (*)

C. One of the locks misadjusted or damaged and/or defective gas cylinder and/or defective R-control cables

156. Keeping all wiring bundles and electrical components free of dust, dirt and contamination to prevent any electrical malfunction due to arcing, short circuit, etc. to comply with?

A. Manufacturer instruction

B. Electrical Wiring Interconnecting Systems (EWIS) maintenance procedure (*)

C. Authority requirements

157. During seat maintenance, to be sure the cleaning agent will not stain or damage the fabric or finish?

A. Read and follow all label directions for the cleaning materials in use

B. Always test a cleaning agent on a small

C. Read and follow all label directions for the cleaning materials in use and always test a cleaning agent on a small (*)

158. If the cushion of passenger seat is thought to have been significantly stained by any organic or inorganic substances?

A. The cushion should be cleaned with suitable cleaning agents.

B. The cushion should be cleaned with suitable cleaning agents, material or device must be used to rubbed or scrubbed the stains on cushion.

C. the cushion should be replaced (*)

159. For light stains on the passenger seat cushions?

A. The cushion should be replaced

B. The cushion should be cleaned with suitable cleaning agents.

C. Dabbing with a soft cloth or sponge and warm water is permissible. (*)

160. During cleaning seatbelt, it must do?

A. Metal components and hardware on seat belts can be cleaned with a lint-free cloth moistened with isopropyl alcohol, with the webbing covered to prevent contact with the isopropyl alcohol. The assemblies must be dried in an area with good air flow and away from sources of heat or sunlight.

B. Webbing can be hand washed using a soft brush and mild soap (e.g., liquid dishwashing soap or household laundry detergent), with the metal parts covered to protect them from contact with the soap.

C. Metal components and hardware on seat belts can be cleaned with a lint-free cloth moistened with isopropyl alcohol, with the webbing covered to prevent contact with the isopropyl alcohol. Webbing can be hand washed using a soft brush and mild soap, with the metal parts covered to protect them from contact with the soap. The assemblies must be dried in an area with good air flow and away from sources of heat or sunlight. (*)

161. To clean plastic item of aircraft seat?

A. Mix liquid soap or plastic cleaner with water to the suggested concentration, depending on the strength needed for cleaning. Apply the solution directly to the plastic and rub with a clean, lint-free cloth to loosen dirt and debris. Wipe with a fresh, clean, lint-free cloth or rinse with water and dry with a turkish towel. (*)

B. Mix liquid soap or plastic cleaner with water to the suggested concentration, depending on the strength needed for cleaning. Apply the solution directly to the plastic and rub abrasive soaps and bleach. Wipe with a fresh, clean, lint-free cloth or rinse with water and dry with a turkish towel.

C. Apply the liquid soap or plastic cleaner directly to the plastic and rub with a clean, lint-free cloth to loosen dirt and debris. Wipe with a fresh, clean, lint-free cloth or rinse with water and dry with a turkish towel.

162. Do not apply dry cleaning of the seat belt assembly, because?

A. Dry cleaning will cause the webbing to lose some of its abrasion-resistant coating and decrease the seat belt service life (*)

B. Dry cleaning will cause the metal components to lose some of its abrasion-resistant coating.

C. Dry cleaning will cause corrosion of the metal components

163. To remove adhesive residue from passenger seat?

A. use a soft cloth wet with small amounts of isopropyl alcohol to remove adhesive residue (e.g., double-sided tape residue). Once the area is clean and free from adhesive residue, use a soft, dry, lint-free cloth to completely dry the area (*)

B. use solvents as a cleaner

C. use non-metal scraper

164. Seat with description "Seat Assembly, LH TPL, In-Arm Foodtray.." means?

A. Seat assembly may has back-mounted foodtrays and in-arm foodtrays (*)

B. Seat assembly may has back-mounted foodtrays

C. Seat assembly may has in-arm foodtrays

165. It does not require immediate replacement of the seat belt if minimal amounts of wear or fraying of the seat belt webbing as follows?

A. The fraying is less than 10% of the width of the webbing (but not at either edge) and less than eight inches in length and less than fifteen broken yarns anywhere other than the edge and less than fifteen torn stitches in a stitch pattern. (*)

B. The fraying is less than 10% of the width of the webbing and less than eight inches in length and less than fifteen broken yarns anywhere other than the edge and less than fifteen torn stitches in a stitch pattern.

C. There is no regulation, it depends on the perception of the maintenance staff.

166. During seat maintenance, the screws, bolts, or other fasteners having stripped threads must be?

A. Replace new (*)

B. Use a plug tap of correct size to clean up threads

C. Clean up slightly rough or burred with a thread file or small three-corner file

167. Seat storage should be maintained in?

A. accordance with seat manufacturer requirements (*)

B. accordance with authorities requirements

C. Normal condition

168. Almost cabin attendant seat can be put in?

A. two positions (*)

B. Only one position

C. three positions

169. The term "Fireblocking" is used to describe?

A. the method or system of providing increased thermal protection for the foam in seat cushions. (*)

B. the method of providing increased thermal protection for the foam in seat cushions.

C. the method or system of providing increased thermal protection for the foam in seat cushions, seat covers.

170. During cabin attendant seat maintenance, if seat pan does not open correctly, that is mainly caused by?

A. Tension spring weakened or broken

B. Articulation seized up

C. Tension spring weakened or broken and/or articulation seized up (*)

171. What is the yellow paper in CMM?

A. Temporary Revision of document (*)

B. Manufacturer Service Letter

C. Normal Revision of document

172. General rules for the insertion of CMM's Temporary Revisions?

A. A yellow page (with higher sequential number) can replace a yellow page and a yellow page cannot replace a white page. (*)

B. A yellow page cannot replace a white page

C. A yellow page (with higher sequential number) can replace a yellow page

173. Related to un-electrical control seat, the fault "seat back will not recline" has main cause as follows?

A. Recline cable assembly loose from actuator end of the hydraulic lock and/or Recline cable out of cable lever slot in seat control assembly and/or Recline cable assembly broken and/or hydraulic lock damaged/inoperative. (*)

B. Recline cable assembly loose from actuator end of the hydraulic lock and/or Recline cable out of cable lever slot in seat control assembly and/or Recline cable assembly broken.

C. Recline cable assembly loose from actuator end of the hydraulic lock and/or Recline cable out of cable lever slot in seat control assembly.

174. If tension of recline cable is not accurately adjusted?

A. the cable simply slips out of the actuator end of the cable assembly or in some case, the control button end of the cable assembly (*)

B. the cable simply slips out of the actuator end of the cable assembly

C. the control button end of the cable assembly

175. What is the signs that the fiberblocking capability of liner system is affected?

A. Wear holes and seams. (*)

B. Discoloration, fading or color transfer from the dress cover to the fireblocking liner and over-limited wear holes and seams.

C. Discoloration, fading or color transfer from the dress cover to the fireblocking liner

176. The aircraft seat fireblocking system includes?

A. the cushions, fireblocking liner and dress cover. (*)

B. the cushions and dress cover.

C. fireblocking liners

177. What are the factors that have an unwanted effect on the function of the fireblocking system?

A. the cleanliness of all the components and the integrity of the fireblocking liner. (*)

B. the cleanliness of all the components and the times of dry cleaning of the fireblocking liner.

C. the cleanliness of all the components and the shelf life of the fireblocking liner.

178. If applicable, the fireblocking liner seams could be repaired by?

A. Using only kevlar thread (*)

B. Using only cotton thread

C. Using only carbon thread

179. To active handicap armrest which equipped on passenger seat?

A. Press the discrete arm latch, lift the front of the arm up, rotating the arm backwards along the side of the back assembly (*)

B. Lift the front of the arm up, rotating the arm backwards along the side of the back assembly

C. Press the discrete arm latch, lift the front of the arm up, rotating the arm forwards along the side of the back assembly.

180. When seat parts can rotate between each other?

A. route the cables parallel to the rotation axis (*)

B. route the cables perpendicular to the rotation axis

C. route the cables circular to the rotation axis

181. To remove minor scratches and nicks on aluminum parts of seatbelt?

A. An abrasive cloth can be used (*)

B. Dry cleaning procedure will be applied

C. the repair is prohibited

182. To remove minor scratches and nicks on steel parts of seatbelt?

A. Crocus cloth can be used (*)

B. Dry cleaning procedure will be applied

C. the repair is prohibited

183. When assembling the seat?

A. replace attaching parts (cotter pins, screws, washers, nuts, grommets, cable holders, etc.) with new parts (*)

B. re-use attaching parts (cotter pins, screws, washers, nuts, grommets, cable holders, etc.)

C. clean and carefully check before re-use attaching parts (cotter pins, screws, washers, nuts, grommets, cable holders, etc.)

184. Seat manufacturer does not support the maintenance of riveted, welded, or bonded assemblies. Because?

A. Repair of these types of assemblies can lead to a loss of structural integrity (*)

B. Repair of these types of assemblies is not economic

C. Repair of these types of assemblies is not reliable

185. If the seat manufacturer does not support the maintenance of riveted, welded, or bonded assemblies. Since these assemblies have very low incidences of failure under normal operating conditions?

A. they are treated as the lowest replaceable assemblies in the maintenance concept for this seat (*)

B. the seat must be unserviceable

C. the seat must be replaceable

186. Layout of Passenger Accommodations or The Aircraft Interior Configuration Document is?

A. an engineering diagram of the aircraft's cabin interior that includes, but is not limited to, locations of passenger and flight attendant seats, emergency equipment, exits, lavatories, and galleys. (*)

B. an engineering diagram of the aircraft's cabin interior that includes, but is not limited to, locations of passenger and flight attendant seats.

C. an diagram of the aircraft's cabin interior that includes, but is not limited to, locations of cockpit; passenger and flight attendant seats, emergency equipment, exits, lavatories, and galleys.

187. When all passenger seats are re-installed, what maintenance data is used to determined the distance (pitch) between seats?

A. Layout of Passenger Accommodations (LOPA) (*)

B. AMM

C. Refer to marks made on seat track.

188. The cabin attendant seats can include?

A. The single cabin-attendant seats; The double cabin-attendant seats; The high-comfort cabin-attendant seats. (*)

B. The mechanical cabin-attendant seats; The electrical cabin-attendant seats

C. The fixed cabin-attendant seats; The movable cabin-attendant seats

189. When you remove of the Floor-Mounted Single Cabin-Attendant-Seat?

A. the O-rings and the seals are discarded. (*)

B. the O-rings and the seals are re-used after cleaning.

C. the O-rings are discarded and the seals are re-used after cleaning.

190. When you remove/install airbag seatbelts, make sure that?

A. the two halves of the seat belt are not closed; there are no magnetic fields near the electronic module. (*)

B. the two halves of the seat belt are closed; there are no magnetic fields near the electronic module.

C. the two halves of the seat belt are closed

191. The seat back may also be restored to the Taxi, Takeoff and Landing (TTOL) position by?

A. activating the recline control button; pushing on the rear of the seat without activating pushing the recline control button. (*)

B. pushing on the rear of the seat without activating pushing the recline control button.

C. activating the recline control button

192. To move any backrest of passenger seat from the upright to the recline position, you must?

A. press the recline control push button and apply pressure against the backrest, release the push button to restrain backrest from any movement (*)

B. press the recline control push button and apply pressure against the backrest

C. press the recline control push button, release the push button to restrain backrest from any movement

193. "WARNING" means?

A. Calls attention to use of materials, processes, methods, procedures or limits which must be followed precisely to avoid injury or death to persons (*)

B. Calls attention to methods and procedures which be followed to avoid damage to equipment

C. Calls attention to methods which make the job easier

194. "CAUTION" is meant?

A. Calls attention to use of materials, processes, methods, procedures or limits which must be followed precisely to avoid injury or death to persons.

B. Calls attention to methods and procedures which be followed to avoid damage to equipment (*)

C. Calls attention to methods which make the job easier

195. "NOTE" is meant?

A. Calls attention to use of materials, processes, methods, procedures or limits which must be followed precisely to avoid injury or death to persons

B. Calls attention to methods and procedures which be followed to avoid damage to equipment

C. All attention to methods which make the job easier (*)

196. You can lock the single door assembly by?

A. a latch in the lavatory (*)

B. a microswitch in the lavatory

C. a number lock in the lavatory

197. When the single door is closed and locked, a microswitch operates?

A. the TOILET CLOSED sign.

B. the TOILET OCCUPIED sign. (*)

C. the TOILET LOCKED sign.

198. When perform cleaning of PC-films?

A. can use MEK

B. should use MEK

C. can not use MEK (*)

199. Behind the washbasin unit door?

A. the water system, the fire extinguisher and the waste box are installed. (*)

B. the oxygen system, the fire extinguisher and the waste box are installed.

C. the fire extinguisher and the waste box are installed.

200. The lavatory is made from?

A. gypsum panels

B. honeycomb panels (*)

C. plastic panels